This specification must be edited for project-specific requirements. It should be reviewed by a qualified civil or geotechnical engineer, architect, or landscape architect familiar with the site conditions.

SECTION 06 73 00 – COMPOSITE DECKING

PART 1 - GENERAL

1.1	SUMMARY

- Section Includes: Composite wood decking approved for direct ground and water content.

- Related Sections:

  - Section 06 11 00 – Wood Framing

1.2	REFERENCES

- Reference Standards:

  - ASTM International (ASTM).

    - ASTM C518 Standard Test Method for Steady-State Thermal Transmission Properties by Means of the Heat Flow Meter Apparatus.

    - ASTM D1037 Standard Test Methods for Evaluating Properties of Wood-Base Fiber and Particle Panel Materials.

    - ASTM D1621 Standard Test Method for Compressive Properties of Rigid Cellular Plastics.

    - ASTM D1761 Standard Test Methods for Mechanical Fasteners in Wood.

    - ASTM D7032 Standard Specification for Establishing Performance Ratings for Wood-Plastic Composite and Plastic Lumber Deck Boards, Stair Treads, Guards, and Handrails.

    - ASTM E84 Standard Test Method for Surface Burning Characteristics of Building Materials.

  - American Wood Council (AWC).

    - AWC NDS National Design Specification for Wood Construction.

  - American Wood Protection Association (AWPA).

    - AWPA E1 Standard Method for Laboratory Evaluation to Determine Resistance to Subterranean Termite.

  - International Codes Council (ICC).

    - ICC- ES AC174 Acceptance Criteria for Deck Board, Span Ratings and Guardrail Systems (Guards and Handrails).

  - US Green Building Council (USGBC).

- LEED V4 LEED (Leadership in Energy and Environmental Design): Green Building Rating System Reference Guide for Building Design and Construction.

1.3	SUBMITTALS

- Product Data: Manufacturer’s standard specifications and descriptive literature, including:

  - Manufacturer’s product technical data sheets.

  - Manufacturer’s installation instructions.

  - Safety Data Sheets (SDS).

- Samples: One 4 inch long decking sample illustrating size, profile, color, and surface finish.

- Certificates: Product certificates signed by manufacturer certifying materials comply with specified performance characteristics and criteria, and physical requirements.

- Warranty: Manufacturer’s published warranty document, including:

  - Manufacturer’s 50-year structural warranty, from date of substantial completion, covering defects in materials.

  - Manufacturer’s [30-Year Elevate] [40-Year Meridian] [50-Year Vision] fade and stain warranty.

- Sustainable Design (LEED) Submittals:

  - LEED Submittals: In accordance with Section [01 35 21 – LEED Requirements].

  - Submit verification for items when appropriate as follows:

    - MR2-A Recycled Content.

    - MR5 Regional Materials.

- Maintenance Data: Manufacturer’s instructions on care and cleaning of composite wood

1.4	QUALITY ASSURANCE

- Installer: Experienced in performing work similar to work of this Section

1.5	DELIVERY, STORAGE & HANDLING

- Deliver materials in accordance with manufacturer’s written instructions.

  - Deliver materials in manufacturer’s original, unopened, undamaged packaging with identification labels intact and product name and manufacturer clearly visible and sizes to suit project.

- Store materials protected from exposure to harmful environmental conditions, clean, dry, frost-free and at recommended temperature and humidity levels.

  - Store composite wood level and flat, off ground or floor, with supports at each end and maximum 24 inches on center.

  - Do not stack composite wood to heights greater than 8 feet high.

  - Cover composite wood with waterproof covering, vented to prevent moisture buildup.

PART 2 - PRODUCTS

2.1 MANUFACTURER

- MoistureShield Inc; 801 Jefferson St, Springdale, AR 72764; Phone: (866) 729-2378,                 (479) 756-400; Email: info@moistureshield.com; Website: www.moistureshield.com

- Substitutions: Not permitted under Division 01

2.2	PERFORMANCE REQUIREMENTS

- Comply with ICC-ES Report VAR-1015.

- Composite Wood Decking: To ASTM D7032 and ASTM D1037.

- Surface Burning Characteristics: To ASTM E84 Class C or Class III.

- Insect Resistance: To AWPA E1.

2.3	MATERIALS

- Decking: Slip, scratch, mold and mildew resistant composite wood decking with high recycled content and approved for direct ground and water contact.

  - Acceptable material: MoistureShield, Inc., [Series Name] [Color Name]

- Decking: Reclaimed wood and polyethylene plastic with additives for coloring and inhibiting fungal and algal growth; free from toxic chemicals.

  - Profiles:

    - Deck board: [1 × 5.4] [2 × 4] [2 × 6] [2 × 8] inches × [12] [16] [20] feet long [with edge grooves for concealed fastening] [with solid edge].

    - Fascia Trim Board: [0.67 × 11.25] inches × 12 feet long.

  - Series: [MoistureShield Series Name].

  - Colors: [MoistureShield Color Name].

  - Recycled content:

    - 95% total recycled content.

2.4	ACCESSORIES

- Mechanical Fasteners: To ASTM D1761.

  - [Carbon steel] [or] [Stainless steel] [composite wood screws] of length in accordance with composite decking manufacturer’s written recommendations.

- Hidden Fasteners:

  - [Carbon steel] [or] [Stainless steel] deck clips capable of elevating deck boards above joists.

    - Acceptable material: MoistureShield hidden fasteners

  - Plastic deck clips with [Carbon steel] [or] [Stainless steel] pre-inserted screw.

    - Acceptable material: MoistureShield hidden fasteners

PART 3 - EXECUTION

3.1	INSTALLER

- Use only installers who have training and experience of work similar to the work of this Section.

3.2	EXAMINATION

- Verification of Conditions: Verify that conditions of substrate previously installed under other Sections or Contracts are acceptable for composite wood decking installation in accordance with manufacturer’s written recommendations.

  - Visually inspect substrate in presence of consultant.

  - Inform consultant of unacceptable conditions immediately upon discovery.

  - Proceed with installation only after unacceptable conditions have been remedied and after receipt of written approval to proceed from Consultant.

  - Starting installation of composite wood decking implies substrate conditions are acceptable for Work of this Section.

3.3	INSTALLATION

- Install composite wood decking in accordance with manufacturer’s written recommendations.

- Do not use composite decking material for structural applications.

- Deck Design: To AWC NDS.

- Layout and install joists at [16 inch maximum on center when decking will be installed perpendicular to joists] [12 inch maximum on center when deck boards will be installed at an angle to joists].

- Do not exceed maximum spans in accordance with manufacturer’s written recommendations.

- Ensure joists are level, plumb and square.

- Install deck boards [in pattern indicated] [perpendicular] [diagonally] to joists [with hidden fastener deck clips] and secure with mechanical fasteners.

  - Use 2 fasteners at each joist support.

  - Allow [3/16] [1/4] inch side-to-side spacing between deck boards.

  - Allow 1/8 inch minimum end-to-end spacing at butt joints between deck boards.

    - Stagger butt joints on decks longer than 20 feet.

    - Stagger butt joints in adjacent rows.

    - Install sister joists to support butt joint ends as needed.

- Cut deck boards true and square to length after securing.

- Install fascia trim boards as indicated.

3.4	CLEANING

- Perform daily progress cleaning.

  - Leave work area clean at end of each day.

- Upon completion, remove surplus materials, rubbish, tools and equipment.

- Collect recyclable waste and dispose of at appropriate recycling facilities.

- Final Cleaning: Clean composite wood surfaces according to manufacturer’s maintenance guidelines.

3.5	PROTECTION

- Protect applied composite wood decking from damage during construction.

- Repair or replace adjacent materials damaged by installation of composite wood decking.

END OF SECTION